Hardware Design Documentation —

Smart Water Purifier

Contents

- MCU selection

- Sensor selection

- Sensor-to-MCU wiring

- Actuator control

- Summary

# MCU Selection

There are various MCUs available for embedded water-quality monitoring systems such as:

- Arduino UNO/Mega

- STM32

- Raspberry Pi Pico W

- ESP32 / ESP32-S3

Among these, ESP32-S3 is selected as the main microcontroller for this application.

## Why ESP32-S3

ESP32-S3 is suitable because it combines wireless connectivity, multiple digital interfaces, ADC/GPIO capability, PWM control, and sufficient processing capability for simultaneously handling water-quality sensors and purification actuators.

### Key reasons:

- Integrated Wi-Fi + Bluetooth LE — enables wireless monitoring and communication without requiring an external communication module.

- Multiple communication interfaces — supports I2C, UART, SPI, GPIO, PWM, and other peripherals required by the sensing and control system.

- Multiple GPIOs — sufficient pins are available for sensors, flow monitoring, valves, pump control, and UV control.

- Sensor-hub role — the ESP32-S3 can collect sensor data locally and make control decisions without continuously depending on an external processor.

- Low-power operation — useful for reducing power consumption when the purifier is idle.

- Cost-effective — combines processing, connectivity, and peripheral interfaces into a single controller.

- Suitable for real-time control — interrupt-based flow measurement and direct GPIO actuator control can be handled by the MCU.

- Expandable architecture — additional sensors or communication peripherals can be added through the available buses.

The ESP32-S3 therefore acts as the central sensing and control unit of the water purifier.

# Sensor Selection

## Water Quality — ADS1115 16-bit ADC

The ADS1115 is used as an external high-resolution ADC for the analog water-quality sensors.

### Interface

- Interface: I2C

- SDA → ESP32-S3 GPIO8

- SCL → ESP32-S3 GPIO9

- VDD → 3.3V buck-converter rail

- GND → common ground

- I2C address: 0x48

- ADDR pin: tied to GND

- Analog Inputs

| ADS1115 Input |  | Sensor |
| --- | --- | --- |
| AIN0 | pH probe |  |
| AIN1 | TDS probe |  |
| AIN2 | Turbidity sensor |  |
| AIN3 | ORP probe |  |

Each sensor is connected through its own signal-conditioning stage before reaching the corresponding ADS1115 input.

### Why ADS1115

The ESP32-S3's internal ADC is not preferred for these measurements because its noise and nonlinearity, particularly near the supply rails, can limit the precision required for measurements such as pH and ORP.

The ADS1115 therefore provides a dedicated higher-resolution acquisition stage for the analog water-quality measurements.

# Electrochemical Sensing — LMP91000

The LMP91000 is used as an electrochemical analog front end for heavy-metal sensing.

### Interface

- Interface: I2C

- SDA/SCL → shared I2C bus

- VDD → 3.3V buck-converter rail

- GND → common ground

- I2C address: 0x4D

The LMP91000 shares the same I2C bus as the ADS1115.

### I2C Addressing

There is no address conflict because:

- ADS1115 → 0x48

- LMP91000 → 0x4D

Therefore, both devices can communicate over the same GPIO8/GPIO9 I2C bus.

### Electrode Connection

The LMP91000 interfaces with a three-electrode electrochemical cell:

- Working electrode

- Reference electrode

- Counter electrode

This provides the analog front-end required for electrochemical heavy-metal sensing.

# Temperature Measurement — DS18B20

The DS18B20 provides digital temperature measurement of the water.

### Interface

- Interface: 1-Wire

- Data → ESP32-S3 GPIO4

- VDD → 3.3V buck-converter rail

- GND → common ground

- Pull-up: 4.7 kΩ between Data and 3.3V

### Why DS18B20

The sensor provides a digital output, avoiding the analog noise problems associated with directly measuring a low-level temperature signal using an MCU ADC.

The 1-Wire interface also requires only a single GPIO, leaving other ESP32-S3 pins available for additional sensors and actuators.

# Flow Measurement — YF-S201

The YF-S201 is used to measure the water flow through the purification system.

### Interface

- Interface: Digital pulse output

- Signal → ESP32-S3 GPIO34

- VCC → 5V buck-converter rail

- GND → common ground

The YF-S201 uses a Hall-effect sensing mechanism to generate pulses corresponding to water flow.

### Interrupt-Driven Measurement

GPIO34 is configured as an interrupt input.

Instead of continuously polling the sensor:

- Water flows through the sensor.

- The Hall-effect sensor generates pulses.

- Each pulse triggers an interrupt.

- The ESP32-S3 counts the pulses.

- Pulse frequency is converted into flow-rate information.

This approach reduces unnecessary CPU polling and allows the controller to respond quickly to changes in water flow.

# Actuator Control

The ESP32-S3 also controls the purification system's actuators.

## Valve Control

Two valves are controlled through dedicated GPIO outputs:

- Valve 1 → GPIO5

- Valve 2 → GPIO6

The GPIO outputs drive the valve coils through MOSFET switching stages.

Each valve coil should have a flyback diode connected across it to suppress the inductive voltage generated when the coil is switched OFF.

### Control arrangement

ESP32-S3 GPIO → MOSFET gate → Valve coil

This prevents the relatively high current required by the valve from being drawn directly from the ESP32-S3 GPIO.

# Pump Control

The purification pump is controlled using:

- Pump PWM control → ESP32-S3 GPIO7

PWM allows the controller to regulate pump operation by adjusting the duty cycle.

This can be used for:

- Pump ON/OFF control

- Flow regulation

- Controlled water circulation

- Protection against abnormal operating conditions

The actual pump power stage should be electrically isolated from the ESP32-S3 GPIO using an appropriate driver/MOSFET stage.

# UV Sterilization Control

The UV sterilization stage is controlled through:

- UV ballast control → ESP32-S3 GPIO15

GPIO15 controls a relay interface, which then switches the UV ballast.

The relay separates the low-voltage ESP32-S3 control circuitry from the higher-power UV ballast circuit.

# Wiring Approach

The water purifier uses a combination of I2C, 1-Wire, interrupt-driven GPIO, ADC, and PWM/GPIO actuator control.

### 1. Shared I2C bus

The ADS1115 and LMP91000 share the same I2C bus:

ESP32-S3 GPIO8 (SDA) ↔ ADS1115 + LMP91000

ESP32-S3 GPIO9 (SCL) ↔ ADS1115 + LMP91000

Their different addresses prevent communication conflicts:

- ADS1115 → 0x48

- LMP91000 → 0x4D

### 2. Analog water-quality sensing

The ADS1115 provides four analog input channels:

- AIN0 → pH

- AIN1 → TDS

- AIN2 → Turbidity

- AIN3 → ORP

Each sensor has an independent signal-conditioning stage before the ADC.

### 3. Temperature measurement

The DS18B20 uses a dedicated 1-Wire connection:

DS18B20 → GPIO4

A 4.7 kΩ pull-up resistor connects the data line to 3.3V.

### 4. Flow measurement

The YF-S201 connects to:

YF-S201 pulse output → GPIO34

GPIO34 operates as an interrupt input, allowing the ESP32-S3 to count flow pulses efficiently.

### 5. Valve control

The two valves use:

- GPIO5 → Valve 1 MOSFET

- GPIO6 → Valve 2 MOSFET

Flyback protection is provided across the valve coils.

### 6. Pump control

GPIO7 provides the PWM control signal for the pump driver.

### 7. UV control

GPIO15 drives the relay interface used to control the UV ballast.

# Power Architecture

The purifier can use separate regulated rails derived from the main power source.

### 3.3V Rail

Supplies the low-voltage sensing and control electronics:

- ESP32-S3

- ADS1115

- LMP91000

- DS18B20

### 5V Rail

Supplies components requiring higher voltage:

- YF-S201 flow sensor

- Other 5V peripherals if required

The actuator power paths for the pump, valves, and UV ballast should be appropriately separated from the sensitive analog measurement circuitry.

A common ground reference is maintained between the low-voltage electronics and sensor interfaces where required.

Summary:

| Layer | Component | Role |
| --- | --- | --- |
| Water-quality sensing | pH, TDS, turbidity, ORP | Measurement of water parameters |
| Analog acquisition | ADS1115 | 16-bit ADC for analog sensor signals |
| Electrochemical sensing | LMP91000 | Heavy-metal sensing AFE |
| Temperature | DS18B20 | Digital water-temperature measurement |
| Flow | YF-S201 | Water-flow measurement |
| Edge MCU | ESP32-S3 | Sensor acquisition and system control |
| Actuation | Valves | Water-path control |
| Actuation | Pump | Water circulation/flow control |
| Sterilization | UV ballast | UV purification control |
| Communication | I2C / 1-Wire / GPIO / PWM | Sensor and actuator interfaces |