OVERVIEW OF SCHEMATICS

POWER SECTION

SENSOR INTERFACES:

I2C — ADS1115 (16-bit ADC for pH, TDS, turbidity, ORP)

- SDA → ESP32-S3 GPIO8

- SCL → ESP32-S3 GPIO9

- VDD → 3.3V (buck converter rail)

- GND → common ground

- I2C address: 0x48 (default, ADDR pin tied to GND)

- AIN0 → pH probe, AIN1 → TDS probe, AIN2 → turbidity sensor, AIN3 → ORP probe — each through its own signal-conditioning stage before reaching the ADC input

- ESP32's built-in ADC: it's noisy and nonlinear near the rails, which defeats the purpose of needing microvolt-level precision for pH/ORP

I2C — LMP91000 (electrochemical AFE, heavy-metal sensing)

- SDA/SCL → shared I2C bus (same lines as ADS1115)

- VDD → 3.3V (buck converter rail)

- GND → common ground

- I2C address: 0x4D (default) — shares the bus with the ADS1115 at a different address, no conflict

- Connects to a 3-electrode cell: working, reference, and counter electrodes

1-Wire — DS18B20 (temperature)

- Data → ESP32-S3 GPIO4

- VDD → 3.3V (buck converter rail)

- GND → common ground

- Requires a 4.7kΩ pull-up resistor between Data and 3.3V

Digital, interrupt-driven — YF-S201 (flow sensor)

- Signal (pulse out) → ESP32-S3 GPIO34

- VCC → 5V (buck converter rail) — Hall-effect sensors need 5V, not 3.3V

- GND → common ground

- Configured as an interrupt input that counts pulses rather than being polled; pulse frequency is proportional to flow rate

GPIO — Actuator outputs (for completeness, since they share the same control unit)

- Valve 1 / Valve 2 control → GPIO5 / GPIO6 (via MOSFET gate, flyback diode across each valve coil)

- Pump control (PWM) → GPIO7

- UV ballast control (relay) → GPIO15